ROOFING SAFETY — TOOLBOX TALK Fall Protection Basics — Your Harness Is Your Life Talk #1 of 30 ¦ Cal/OSHA: §1669–1672, §1730–1731 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Roofer Falls 18 Feet — No Harness

A 34-year-old roofer in San Bernardino was replacing shingles on a residential roof when he stepped backward off the edge. He was not wearing a harness. He fell 18 feet onto a concrete driveway, suffering a broken pelvis, shattered wrist, and traumatic brain injury. His crew said they had a harness in the truck — they just didn't think they needed it on 'a small job.' The company was fined $87,400.

Discussion starter: What would have happened differently if he had been wearing his harness that morning?

KEY SAFETY POINTS — TODAY'S TOPIC

When Is Fall Protection Required?

- 7.5 feet or higher — commercial/non-residential roofing (Cal/OSHA §1670)

- 6 feet or higher — all residential roofing operations (Cal/OSHA §1731)

- ANY height — steep slopes steeper than 7:12 (30 degrees)

- ANY height — working near a skylight, roof opening, or unprotected hole

- There is NO 'small job' exception to fall protection requirements

Your Three Options for Fall Protection

- Personal Fall Arrest System (PFAS): full-body harness + lanyard + anchor point

- Guardrail system: minimum 42-inch top rail, 21-inch mid-rail, 4-inch toe board

- Warning line system: only on low-slope roofs — lines must be 5 feet minimum from edge

Inspecting Your Harness Before Every Use

- Check all straps for cuts, fraying, or burns — if in doubt, take it out of service

- Inspect buckles, D-rings, and snap hooks for cracks, corrosion, or deformation

- Check the lanyard for wear, cuts, and that the energy absorber is NOT deployed

- Verify anchor point integrity before clipping in — 5,000 lbs minimum per worker

- Never use a harness that has caught a fall — replace immediately, no exceptions

CAL/OSHA REQUIREMENT: Cal/OSHA §1670 requires fall protection at 7.5 ft (commercial) and 6 ft (residential roofing). Personal Fall Arrest Systems must be anchored to points capable of supporting 5,000 lbs per worker.

DISCUSSION QUESTIONS — ASK YOUR CREW

- What fall protection method are we using on this job site today? Is it set up correctly?

- Have you ever seen someone skip wearing fall protection? What happened?

- Where are the designated anchor points on this job site? Are they rated for 5,000 lbs?

- What do you do if you notice a harness is damaged before a shift?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | At what height is fall protection required for residential roofing in California? | 6 feet |
| 2 | What is the minimum load rating for a fall arrest anchor point? | 5,000 lbs per worker |
| 3 | Can you use a harness that has caught a previous fall? | No — replace immediately |
| 4 | What are the three acceptable fall protection methods for roofing? | PFAS, guardrail, warning line |
| 5 | On a slope steeper than 7:12, what fall protection is required? | PFAS (harness) at any height |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #1: Fall Protection Basics — Your Harness Is Your Life  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years