ROOFING SAFETY — TOOLBOX TALK Roof Openings & Skylights — The Invisible Killers Talk #2 of 30 ¦ Cal/OSHA: §1669–1672, §1730–1731 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Worker Falls Through Skylight — Fatal

A 28-year-old roofer in Los Angeles stepped onto what appeared to be a solid fiberglass skylight while walking across a commercial roof. The skylight shattered under his weight. He fell 24 feet to a concrete floor below. He died at the scene. OSHA investigators noted the skylight cover was UV-degraded and appeared identical to the surrounding roofing material. The employer had not marked or guarded any of the six skylights on the roof.

Discussion starter: How do you identify skylights and openings on a roof you've never worked on before?

KEY SAFETY POINTS — TODAY'S TOPIC

Identifying Roof Openings Before You Step

- Walk the entire roof perimeter and surface before beginning work — identify ALL openings

- Skylights may be hidden under roofing felt, debris, or look like solid roof panels

- Never assume a skylight or cover can hold your weight — always treat as an opening

- Mark ALL openings with bright spray paint, flagging tape, or signs before work begins

Guarding Roof Openings — Your Three Options

- Cover: must support 200 lbs, be secured against displacement, and labeled 'HOLE — DO NOT REMOVE'

- Guardrail: 42-inch top rail, 21-inch mid-rail, 4-inch toe board around entire opening

- PFAS: full-body harness with anchor away from the opening

- Verbal warnings and yellow tape are NOT acceptable as the sole protection

Skylight-Specific Rules

- Fiberglass and plastic skylights become brittle and discolored with age — NEVER step on them

- Install a skylight screen or guardrail before any worker walks near the skylight

- When in doubt, treat every transparent or translucent roof panel as an unprotected opening

- On commercial roofs, count and mark skylights during your site survey before tools come out

CAL/OSHA REQUIREMENT: Cal/OSHA §1632 requires all floor and roof openings to be guarded with a cover, guardrail, or personal fall arrest system at all times. Skylight covers must be capable of supporting 200 lbs without failure.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Did anyone walk the roof before we started today to identify openings? What did you find?

- How do we mark skylights on this job so the whole crew knows where they are?

- What do you do if you find a skylight that is not marked or protected?

- Have you ever been on a roof where skylights were not obvious? How did you handle it?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What must a roof opening cover be capable of supporting? | 200 lbs minimum |
| 2 | Can you use yellow caution tape alone to guard a roof opening? | No — not acceptable as sole protection |
| 3 | What should you do before work begins on a roof with skylights? | Survey and mark/guard all skylights |
| 4 | Should you step on a fiberglass skylight if it looks solid? | Never — treat as an unprotected opening |
| 5 | What label is required on a temporary hole cover? | HOLE — DO NOT REMOVE |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #2: Roof Openings & Skylights — The Invisible Killers  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years