ROOFING SAFETY — TOOLBOX TALK Steep Slope Roofing — When the Pitch Gets Serious Talk #3 of 30 ¦ Cal/OSHA: §1669–1672, §1730–1731 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Roofer Slides Off Steep-Slope Tile Roof

A tile roofer in Riverside was working on a 9:12 slope roof without a harness because his foreman told him 'the felt gives you enough grip.' He was carrying a bundle of tiles when he lost his footing and slid 22 feet off the edge. He fell into a landscaping bed, breaking both ankles and three vertebrae. He will never roof again. The company paid $134,000 in fines and $1.1 million in workers' compensation costs.

Discussion starter: What does your company require on steep slopes right now? Is that the same as what Cal/OSHA requires?

KEY SAFETY POINTS — TODAY'S TOPIC

Understanding Slope and When PFAS Is Mandatory

- Slope 7:12 (30 degrees) or steeper: PFAS required at ANY height, no exceptions

- Slope 4:12 to 7:12: standard fall protection applies at trigger height (6 or 7.5 ft)

- Slope 0:12 to 4:12 (low slope): warning line system permitted at proper distances

- Know your slope before you start — measure rise over run if you're unsure

Roof Jacks & Scaffold Planks on Steep Slopes

- Roof jacks must be secured with nails through the factory-punched holes only

- Never pry a roof jack off — remove the nails properly when moving down the roof

- Scaffold planks must be full-width and extend minimum 6 inches past each jack

- Do not stand on the ends of scaffold planks — stay between the jacks

- Inspect roof jacks and planks before installation — no cracks, splits, or corrosion

Working Safely on Steep Slopes

- Keep three points of contact when moving across steep slopes

- Never run materials up a steep slope without a hoist or lifeline

- Work from a secured plank system — never from the bare felt or shingles alone

- Wear non-slip roofing boots — inspect soles for wear before every shift

- Clear all debris and cut-off pieces from your work area continuously

CAL/OSHA REQUIREMENT: Cal/OSHA §1731 and §1670 require a Personal Fall Arrest System (PFAS) on all roof slopes exceeding 7:12 regardless of height. Roof jacks and planks must be secured to ridge boards with nails through holes — never pried.

DISCUSSION QUESTIONS — ASK YOUR CREW

- What slope is this roof? Does that trigger PFAS requirements?

- Are our roof jacks installed correctly with nails through the holes — not pried?

- What footwear is everyone wearing today? Are the soles in good condition?

- If you feel unstable on a steep slope, what should you do immediately?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | At what slope is a PFAS (harness) required regardless of height? | 7:12 (30 degrees) or steeper |
| 2 | How should roof jacks be secured to the roof? | Nails through factory-punched holes only |
| 3 | How far must scaffold planks extend past each roof jack? | Minimum 6 inches |
| 4 | Is a warning line system acceptable on a 9:12 slope? | No — PFAS required at any height |
| 5 | What type of footwear should roofers wear on steep slopes? | Non-slip roofing boots with good soles |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #3: Steep Slope Roofing — When the Pitch Gets Serious  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years