ROOFING SAFETY — TOOLBOX TALK Ladder Inspection — Catch Problems Before They Catch You Talk #5 of 30 ¦ Cal/OSHA: §1675, §1675.2 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Cracked Rung Breaks Under Load — Worker Falls

A roofer in Fresno climbed a fiberglass extension ladder that had a hairline crack in the third rung from the bottom. The crack had been there for weeks — everyone on the crew had noticed it but nobody removed the ladder from service. The rung snapped when he stepped on it with a bundle of shingles on his shoulder. He fell backward 8 feet and landed on the back of his head. He suffered a skull fracture and was hospitalized for 3 weeks.

Discussion starter: Does your crew have a system for removing defective ladders from service? Would someone actually remove a ladder if they noticed a defect?

KEY SAFETY POINTS — TODAY'S TOPIC

What to Look For When Inspecting a Ladder

- Side rails: cracks, bends, dents, corrosion, or loose/missing rivets

- Rungs and steps: cracks, bends, loose connections, slippery surface, missing anti-slip coating

- Ladder feet: missing, cracked, or worn rubber feet — feet must contact the ground fully

- Extension hardware: broken or bent hooks, worn rung locks, frayed rope on extension ladders

- Fiberglass: check for delamination, exposed fibers, or UV damage (chalky/faded appearance)

Removing Defective Ladders — No Exceptions

- If you find a defect: remove the ladder from service immediately

- Tag it 'DO NOT USE — DEFECTIVE' with tape or a tag

- Report to your supervisor so it can be repaired or replaced

- Never use electrical tape, duct tape, or wire to 'fix' a broken rung or cracked rail

- A $300 replacement ladder is always cheaper than a workers' comp claim

Ladder Storage and Maintenance

- Store ladders horizontally on racks — never leaned against a wall unsupported

- Keep ladders out of direct sunlight when not in use (UV damages fiberglass)

- Clean ladder feet and rungs of dirt, mud, and tar after every job

- Never paint wooden ladders — paint hides cracks and defects

- Inspect ladders after they have been dropped, fallen, or involved in any incident

CAL/OSHA REQUIREMENT: Cal/OSHA §1675 requires all ladders to be inspected before use. Ladders with cracks, broken rungs, missing feet, bent side rails, or damaged hardware must be immediately removed from service and tagged 'DO NOT USE.'

DISCUSSION QUESTIONS — ASK YOUR CREW

- When was the last time anyone inspected our ladders? Who is responsible for that?

- Do we have a 'DO NOT USE' tag system on this job site?

- What would you do if your supervisor told you to use a ladder you thought was defective?

- How many ladders do we have on this job? Have they all been inspected recently?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What should you do immediately when you find a cracked rung on a ladder? | Remove from service and tag DO NOT USE |
| 2 | Can you repair a cracked ladder rail with duct tape? | No — must be removed from service |
| 3 | Why should you never paint a wooden ladder? | Paint hides cracks and defects |
| 4 | When must you inspect a ladder after an incident? | Immediately after it is dropped, falls, or involved in any incident |
| 5 | What does UV damage look like on a fiberglass ladder? | Chalky, faded, or discolored appearance |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #5: Ladder Inspection — Catch Problems Before They Catch You  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years