ROOFING SAFETY — TOOLBOX TALK Ladder Placement at the Eave — Getting On and Off Safely Talk #6 of 30 ¦ Cal/OSHA: §1675, §1675.2 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Worker Falls Getting Off Roof — Missed the Ladder

At the end of a shift, a roofer in Orange County stepped backward off the eave to find the ladder and missed it by 18 inches. The ladder had shifted during the day. He fell 14 feet to a concrete walkway. Both his feet and ankles were shattered. He required multiple surgeries and was out of work for 14 months. The crew had no designated ladder access point — workers came and went wherever they found the ladder.

Discussion starter: What is your system for getting on and off the roof safely at the end of a long, tired day?

KEY SAFETY POINTS — TODAY'S TOPIC

Establishing a Safe Ladder Access Point

- Designate ONE ladder access point per roof section at the start of every job

- Mark it clearly with a flag, cone, or paint so all crew members know where it is

- The ladder must be secured top AND bottom before anyone uses it

- Keep the area around the base and top of the ladder clear of materials and debris

- Brief the whole crew on the access point location at the daily tailgate meeting

Getting On and Off the Roof Safely

- Always face the ladder — never step backward off the eave onto a ladder

- Grip both side rails (not the rungs) when transferring from roof to ladder

- Come to a complete stop at the eave before beginning descent — don't rush

- At the top: kneel or crouch, grab both rails, then swing onto the ladder facing it

- At the end of the day when tired: slow down — more falls happen in the last hour of work

Ladders in Windy Conditions

- Check ladder stability more frequently in wind — re-secure if movement is felt

- Do not use ladders in winds exceeding 25 mph or when gusts make balance difficult

- A second person should foot the ladder in windy conditions

- Never leave an unsecured ladder unattended where wind could topple it

CAL/OSHA REQUIREMENT: Cal/OSHA §1675 requires ladders to be placed so workers can safely transition to and from the roof surface. Workers must face the ladder when dismounting. A designated ladder access point must be established at each job site.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Where is our designated ladder access point on this job? Does everyone know it?

- How do you safely get off the roof when you are tired at the end of a long day?

- Have you ever had a ladder shift while you were trying to get on or off the roof?

- What do we do with ladders during strong winds or at the end of the day?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | How should you face when descending a ladder from the roof? | Facing the ladder (never step backward off the eave) |
| 2 | How many ladder access points should be designated per roof section? | One designated, clearly marked point |
| 3 | What should you grab when transferring from the roof to the ladder? | Both side rails — not the rungs |
| 4 | When during the workday do most ladder falls occur? | The last hour — when workers are tired |
| 5 | What wind speed should stop ladder use? | 25 mph or when gusts affect balance |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #6: Ladder Placement at the Eave — Getting On and Off Safely  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years