ROOFING SAFETY — TOOLBOX TALK Hot Tar PPE — Protecting Yourself From Burns Talk #11 of 30 ¦ Cal/OSHA: §1940–1941, §1505 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Hot Tar Spill Burns Roofer's Legs Severely

A roofer in San Diego was carrying a bucket of hot tar up a ladder when he lost his grip on the second rung and spilled the entire bucket down his legs. He was wearing shorts. The tar — heated to 400°F — adhered to his skin and caused third-degree burns over 40% of his lower body. He spent 6 weeks in a burn unit and required 11 skin graft surgeries. He lost the use of his right leg. His crew said they 'always worked in shorts' because of the heat.

Discussion starter: Have you ever seen workers on a roofing site wearing shorts while handling hot tar? What's the right approach?

KEY SAFETY POINTS — TODAY'S TOPIC

Mandatory PPE for Hot Tar Operations

- Gloves: must fit snugly at the wrists — no gaps where tar can enter

- Upper body: long sleeves or appropriate protection — no bare arms or no shirt

- Lower body: long pants — never shorts when handling or near hot tar

- Footwear: high-top leather boots that cover the ankle — never open shoes

- Eye protection: safety glasses or face shield when ladling or pouring hot tar

Carrying Hot Tar Safely

- Use a bucket with a proper bail handle — never carry by hooking your fingers under the rim

- Do not carry anything in your other hand while carrying a hot tar bucket

- Never run while carrying hot tar — walk deliberately with full attention to your footing

- Do not carry hot tar up a ladder in a bucket — use a hoist or pitch mop system

- When handing off buckets, communicate clearly: 'Hot tar coming' before passing

Burns — Immediate First Aid

- DO NOT try to remove clothing stuck to a burn — you will pull off skin

- Cool with cool (not ice cold) running water for minimum 20 minutes

- Do not apply butter, oil, or any household remedy

- Cover loosely with a clean bandage — do not wrap tightly

- Call 911 for any burn larger than the palm of the worker's hand or involving face/hands/genitals

CAL/OSHA REQUIREMENT: Cal/OSHA §1505 and the Cal/OSHA Construction Safety Orders require workers handling hot tar to wear close-fitting gloves, long-sleeved protection, and appropriate footwear. No worker shall handle hot tar without a shirt or appropriate upper body protection.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Is everyone on the crew wearing the correct PPE for hot tar work today?

- How do we safely move hot tar from the kettle to the roof on this job?

- Does everyone know the first aid steps for a hot tar burn?

- Where is the nearest emergency burn treatment facility from this job site?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What type of gloves are required for hot tar work? | Gloves that fit snugly at the wrists — no gaps |
| 2 | Is it acceptable to carry a hot tar bucket up a ladder by hand? | No — use a hoist or pitch mop system |
| 3 | What should you do if clothing sticks to a burn? | Do NOT remove it — cool with water and call 911 |
| 4 | For how long must you cool a burn with running water? | Minimum 20 minutes |
| 5 | What body protection is required below the waist during hot tar operations? | Long pants — never shorts |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #11: Hot Tar PPE — Protecting Yourself From Burns  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years