ROOFING SAFETY — TOOLBOX TALK Torch Work & Open Flame — Preventing Roof Fires Talk #12 of 30 ¦ Cal/OSHA: §1940–1941 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Torch Ignites Roofing Felt — Building Burns Down

A commercial roofer in Oakland was applying torch-down modified bitumen on a 30,000 sq ft roof. The torch flame ignited dry felt at a parapet wall base. The roofer didn't notice because he was moving forward. A fire watch had not been assigned — it wasn't standard practice for the crew. By the time the fire was spotted, it had spread under the roof membrane. The building, a fully occupied warehouse, suffered $4.2 million in damage. No fire watch was in place.

Discussion starter: On your last torch job, was a dedicated fire watch in place for 30 minutes after the torch was done?

KEY SAFETY POINTS — TODAY'S TOPIC

Fire Watch Requirements

- A dedicated fire watch must be in place during ALL torch work and for 30 minutes after

- The fire watch person does nothing else — their only job is watching for fire

- Fire watch must have: a working fire extinguisher and a phone to call 911

- Fire watch must have unobstructed visibility of the work area

- When torch work moves to a new area, fire watch continues at the previous area for 30 minutes

Safe Torch Operation

- Inspect hose, regulator, and connections for leaks before igniting — use soapy water

- Never use an open flame to check for gas leaks

- Keep the flame moving — never hold the torch on one spot longer than 2–3 seconds

- Keep torches away from: wood blocking, felt underlayment, adhesives, and solvents

- Extinguish the torch properly when moving, taking breaks, or going down to the ground

Storing and Transporting Propane

- Store propane cylinders upright and secured against tipping at all times

- Never store propane cylinders in enclosed vehicles or trailers

- Keep cylinders away from heat sources, open flames, and direct sunlight

- Inspect cylinders and hose connections before each use

- Follow the '10-foot rule': no ignition sources within 10 feet of propane storage

CAL/OSHA REQUIREMENT: Cal/OSHA §1940 requires a fire watch during all torch-applied roofing operations and for a minimum of 30 minutes after torch work is complete. A fire watch person must have a fire extinguisher and be dedicated solely to watching for fires.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Do we have a designated fire watch person assigned for today's torch work?

- How do we verify that hose connections are leak-free before lighting the torch?

- Where are our propane cylinders stored overnight — is that location safe?

- If you were the fire watch and you spotted a small fire starting, what would you do first?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | How long must a fire watch continue after torch work is complete? | Minimum 30 minutes |
| 2 | Can the torch operator serve as their own fire watch? | No — fire watch must be a dedicated separate person |
| 3 | How do you check propane hose connections for leaks? | Soapy water — never an open flame |
| 4 | Where must propane cylinders be stored? | Upright, secured, away from heat and enclosed spaces |
| 5 | How long should you hold the torch on one spot during application? | No longer than 2–3 seconds — keep it moving |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #12: Torch Work & Open Flame — Preventing Roof Fires  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years