ROOFING SAFETY — TOOLBOX TALK Hard Hats & Head Protection — More Than a Rules Requirement Talk #13 of 30 ¦ Cal/OSHA: §3380–3385 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Falling Shingle Fragment Kills Unprotected Ground Worker

A laborer on a roofing job in Stockton was cleaning up debris on the ground while the roofing crew worked above. A bundle of shingles was being slid off the roof and a single shingle broke free, tumbling 22 feet and striking the laborer on the back of the head. He was not wearing a hard hat. He died from blunt force trauma. The roofing crew had hard hats in the truck — there were not enough for the ground crew because 'they're just doing cleanup.'

Discussion starter: Does your ground crew wear hard hats? Is there a policy about it, or is it left to individual choice?

KEY SAFETY POINTS — TODAY'S TOPIC

Hard Hat Classes — Use the Right One

- Class G (General): protects against impact and low-voltage electrical contact (up to 2,200V)

- Class E (Electrical): protects against impact and higher-voltage electrical contact (up to 20,000V) — required near power lines

- Class C (Conductive): protects against impact only — no electrical protection

- For most roofing work: Class E is the safest choice and covers all scenarios

Wearing and Maintaining Your Hard Hat

- Wear the hard hat with the brim forward — do not wear it backward unless specifically designed for reverse wear

- Adjust the suspension so the hat sits 1 inch above the forehead

- Inspect the shell for cracks, dents, and UV damage before each use

- Inspect the suspension for fraying, tears, and loose connections

- Never store a hard hat in direct sunlight (UV degrades the shell) or in the rear window of a truck

When Hard Hats Are Required on the Roof

- All ground workers when crew is working overhead — no exceptions

- All workers when loading materials or working near edges where objects could fall from above

- When working near overhead obstructions (pipes, rafter tails, eave brackets)

- Hard hat must be replaced after any significant impact — even if it looks undamaged (the shell absorbs impact invisibly)

CAL/OSHA REQUIREMENT: Cal/OSHA §3381 requires hard hats on construction sites where there is a risk of head injury from falling objects, bumping into objects, or electrical shock. Class E hard hats are required near electrical hazards.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Is everyone on this job wearing the correct class of hard hat for the hazards present?

- Do our ground workers have hard hats? Where are they stored?

- When was the last time anyone replaced their hard hat? Do you know when yours is due?

- Have you ever had your hard hat hit something? Did you replace it afterward?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What class of hard hat is required when working near energized power lines? | Class E |
| 2 | How far above the forehead should a hard hat sit? | 1 inch |
| 3 | Why should you replace a hard hat after a significant impact even if it looks fine? | The shell absorbs impact energy invisibly — structural integrity is compromised |
| 4 | Can you store a hard hat in the rear window of your truck? | No — UV and heat degrade the shell |
| 5 | Who on the job site must wear a hard hat when roofing work is happening overhead? | ALL workers, including ground crew |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #13: Hard Hats & Head Protection — More Than a Rules Requirement  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years