ROOFING SAFETY — TOOLBOX TALK Housekeeping — The Foundation of a Safe Job Site Talk #19 of 30 ¦ Cal/OSHA: §1512, §1513 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Worker Trips on Scrap Shingles — Falls Off Roof Edge

An experienced roofer in Riverside tripped over a pile of scrap shingles left near the roof edge at the end of a long day. The pile had been there since mid-morning. His momentum carried him to the edge where he managed to grab the gutter — which gave way under his weight. He fell 16 feet and fractured his spine. 'The scrap pile had been there all day. I knew it was there. I just wasn't paying attention at 4 PM when I was tired.' The company was cited $18,500 for failure to maintain clear walkways.

Discussion starter: Walk the roof in your mind: is there anything on this roof right now that someone could trip over?

KEY SAFETY POINTS — TODAY'S TOPIC

Keep the Roof Clear — Throughout the Day

- Clear all scrap material and cut-offs from the work area continuously — not just at end of day

- Establish a designated scrap pile area away from walkways, edges, and skylights

- Remove cut nails and staples from walking surfaces — they cause punctures and slips

- Stack materials neatly and away from roof edges — no stacks within 3 feet of unprotected edges

- Remove debris from ladders and roof access points before the end of every shift

Ground Level Housekeeping

- Barricade the ground area below the work zone — protect pedestrians from falling debris

- Post warning signs at the ground perimeter: 'ROOFING IN PROGRESS — HARD HAT AREA'

- Do not throw materials, debris, or tools off the roof — use a chute or lower by hand

- Keep the area around the ladder base clear — materials near the base create trip hazards

- Dispose of waste in designated containers — never leave shingles, wrapping, or tar paper scattered

End-of-Day Cleanup Routine

- No worker leaves until the work area is cleared and materials are stacked or removed

- Account for all tools before leaving — missing tools create hazards for the next day

- Secure loose materials against wind overnight — stack and brace or remove from roof

- Confirm all ladder access points are secured

- Brief the crew: what's staying on site overnight, what hazards remain, what to address first tomorrow

CAL/OSHA REQUIREMENT: Cal/OSHA §1513 requires construction sites to be maintained in a clean and orderly condition. Passageways, walkways, and stairways must be kept clear of tripping hazards and debris at all times.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Walk the roof right now mentally — what debris or hazards are present that we should address today?

- What is our end-of-day cleanup process on this job? Is everyone clear on their responsibilities?

- Are we protecting pedestrians below from falling debris? What's in place right now?

- Who is responsible for cleanup at the end of the shift — the whole crew or specific individuals?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | How often should scrap material be cleared from the roof work area? | Continuously throughout the day — not just at end of shift |
| 2 | How close to an unprotected roof edge can you stack materials? | No closer than 3 feet |
| 3 | Is it acceptable to throw old shingles off the roof edge if pedestrians are cleared? | No — use a chute or lower by hand |
| 4 | What must be done to materials left on the roof overnight? | Secured against wind — stacked, braced, or removed |
| 5 | What must be posted at the ground perimeter during roofing work? | Warning signs: 'ROOFING IN PROGRESS — HARD HAT AREA' |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #19: Housekeeping — The Foundation of a Safe Job Site  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years