ROOFING SAFETY — TOOLBOX TALK Material Handling & Storage — Preventing Strains and Collapses Talk #20 of 30 ¦ Cal/OSHA: §1512, §1513 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Bundle of Shingles Slides Off Roof — Strikes Ground Worker

A bundle of shingles was stacked near the eave of a residential roof in San Jose for end-of-day pickup. Overnight, it shifted and slid off the roof, landing on a homeowner's car and then striking a pedestrian walking by on the sidewalk. The pedestrian suffered a broken collarbone and filed a lawsuit against the roofing contractor. Cal/OSHA cited the company for improper material storage. The settlement cost the contractor $240,000.

Discussion starter: How do you safely stage materials on a roof when you're coming back to finish the job tomorrow?

KEY SAFETY POINTS — TODAY'S TOPIC

Safe Material Staging on the Roof

- Never stage materials within 3 feet of an unprotected roof edge

- Distribute weight evenly — do not stack more than one bundle high near the ridge without bracing

- On slopes: use toe boards, cleats, or material staging boards to prevent sliding

- Materials left overnight must be secured — strapped, stacked flat, or braced against movement

- Check manufacturer's guidelines for maximum load before staging bundles on the deck

Proper Lifting Technique — Protect Your Back

- Shingle bundles weigh 50–80 lbs — use team lifting for heavy loads

- Lift technique: feet shoulder-width apart, bend at the knees, keep the load close, lift with your legs

- Never twist while lifting — pivot your whole body instead

- Use mechanical assists when available: conveyors, boom loaders, and material hoists

- Stretch before the shift — back injuries peak in the first 2 hours of work when muscles are cold

Material Hoisting Safety

- Inspect hoist, rope, and hooks before each use

- Never stand directly below a load being hoisted

- Establish a drop zone and keep all workers out of it during hoisting operations

- Communicate clearly between the ground and roof crew before beginning any lift

- Never exceed the rated capacity of the hoist — read the label

CAL/OSHA REQUIREMENT: Cal/OSHA requires safe material handling practices to prevent musculoskeletal injuries, the most common non-fatal injury in construction. Roofing materials must be stored to prevent sliding, tipping, or falling.

DISCUSSION QUESTIONS — ASK YOUR CREW

- How are materials staged on this roof right now? Are they within 3 feet of any edge?

- Is anyone on this crew dealing with back pain from lifting? What can we do differently?

- Do we have a material hoist available? Are we using it consistently?

- What will happen to materials on this roof overnight — are they secured?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What is the minimum setback for staged materials from an unprotected roof edge? | 3 feet |
| 2 | What is the correct lifting technique to protect your back? | Bend knees, keep load close, lift with legs, do not twist |
| 3 | When should team lifting be used on a roofing job? | For any load over ~50 lbs, such as shingle bundles |
| 4 | Where should workers stand during a material hoisting operation? | Never below the load — stay out of the drop zone |
| 5 | When must materials staged on a slope be secured? | Always — use toe boards, cleats, or bracing to prevent sliding |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #20: Material Handling & Storage — Preventing Strains and Collapses  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years