ROOFING SAFETY — TOOLBOX TALK Solar Roofing Electrical Hazards — The New Danger on the Roof Talk #24 of 30 ¦ Cal/OSHA: §2940.3, §1600 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Roofer Shocked by Solar Conduit During Re-Roof

A roofing contractor in San Jose was re-roofing a home that had a solar panel system installed 5 years earlier. While removing old shingles near the conduit run, a worker cut through a DC wiring conduit with a roofing hook. He received a significant DC shock. The homeowner had told the crew the 'solar was turned off' — but the inverter being off does NOT de-energize the DC wiring between panels and the inverter. The worker survived but required hospitalization.

Discussion starter: Have you worked on roofs with existing solar installations? Did you treat the wiring as live?

KEY SAFETY POINTS — TODAY'S TOPIC

Why Solar Roofs Are Different

- Solar panels generate DC electricity any time light hits them — even on cloudy days

- Turning off the inverter or the disconnect does NOT de-energize the DC wiring between panels

- DC current from solar panels can be 300–600 volts — extremely dangerous

- The only safe assumption: ALL solar conduit and wiring on the roof is LIVE at all times

- Never cut, drill, or drive fasteners near solar conduit or junction boxes without verification

Before Working on a Solar-Equipped Roof

- Identify all conduit runs, junction boxes, and wiring paths before beginning work

- Contact the solar installer or a licensed electrician to advise on safe work procedures

- Do not attempt to disconnect or modify any solar components — licensed electricians only

- If solar panels must be temporarily removed for re-roofing: hire a licensed solar contractor

- Document the presence of solar on your job hazard analysis before work begins

Protecting Your Crew on Solar Jobs

- Brief the entire crew on the solar hazard before the first nail is pulled

- Mark all conduit runs with tape or chalk so the whole crew knows where they are

- Use non-conductive tools when working near wiring

- If any wiring appears damaged or exposed: stop work and call a licensed electrician

- Homeowner's 'the solar is off' does not mean the roof wiring is safe

CAL/OSHA REQUIREMENT: Cal/OSHA requires electrical safety procedures when working near or on roofs with solar panel installations. Solar panels generate DC current whenever light is present — they cannot be fully de-energized by shutting off the inverter.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Have any of you worked on a re-roof with existing solar panels? What did you do differently?

- Does this job site have solar? Where is the conduit running across the roof?

- How would you handle a homeowner who tells you the solar is turned off and it's safe to work?

- Do we have a procedure in our IIPP for working on solar-equipped roofs?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | Does turning off the solar inverter make the roof wiring safe to work around? | No — DC wiring between panels and inverter remains live |
| 2 | What voltage can DC wiring on a residential solar system carry? | 300–600 volts |
| 3 | Who should disconnect or remove solar panels before a re-roof? | A licensed solar contractor — never the roofing crew |
| 4 | How should all solar conduit runs be marked before work begins? | With tape or chalk so the entire crew knows their location |
| 5 | Can solar panels generate electricity on a cloudy day? | Yes — any light hitting panels generates DC current |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #24: Solar Roofing Electrical Hazards — The New Danger on the Roof  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years