ROOFING SAFETY — TOOLBOX TALK Scaffold Safety — Building It Right the First Time Talk #25 of 30 ¦ Cal/OSHA: §1635–1655 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Unsupported Scaffold Collapses — Two Workers Fall

Two roofers in Los Angeles were working from a scaffold system that had not been tied to the building because 'it was just for a day.' When one worker moved a bundle of tiles to the platform, the weight shift caused the unsecured scaffold to rotate and collapse. Both workers fell 14 feet. One broke his femur and hip; the other broke three ribs and punctured his lung. Cal/OSHA found the scaffold was not tied to the building, had no toe boards, and was erected by workers with no scaffold training. Fines totaled $87,000.

Discussion starter: Has anyone on this crew been trained in scaffold erection? Who erected the scaffold on this job?

KEY SAFETY POINTS — TODAY'S TOPIC

Who Can Erect a Scaffold?

- Scaffold must be erected by or under the direction of a qualified person

- A 'qualified person' has scaffold-specific training — not just general construction experience

- Before using any scaffold: have a competent person inspect it

- Do not modify a scaffold after it has been inspected — any change requires re-inspection

- Document who erected the scaffold and that it was inspected before use

Scaffold Load Requirements

- All scaffolds must be capable of supporting 4× the maximum intended load

- Never exceed the load rating — check the manufacturer's label before loading

- Distribute weight evenly across the platform — no concentrated loading at one end

- Do not stack materials beyond what is needed for immediate work

- Include the weight of workers, tools, AND materials in your load calculation

Required Scaffold Components

- Top rail: 42 inches above the platform

- Mid-rail: halfway between top rail and platform

- Toe board: minimum 4 inches high — prevents tools and materials from falling onto workers below

- Platform: must be fully planked with no gaps larger than 1 inch between planks

- Tying and bracing: scaffolds over 4:1 height-to-base ratio must be tied to the building

CAL/OSHA REQUIREMENT: Cal/OSHA §1637 requires all scaffolds to be designed and erected by or under the supervision of a qualified person. Scaffolds must support 4 times the maximum intended load. All platforms must have guardrails, mid-rails, and toe boards.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Who erected the scaffold on this job? Were they qualified to do so?

- Has a competent person inspected the scaffold today? When?

- What is the load rating of this scaffold? Are we within the limit with our crew and materials?

- Does the scaffold have all required components — guardrails, mid-rail, toe boards, and full planking?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What load capacity must all scaffolds be rated for? | 4 times the maximum intended load |
| 2 | What is the required height of scaffold top rails? | 42 inches above the platform |
| 3 | What is the minimum height of a scaffold toe board? | 4 inches |
| 4 | What is the maximum gap allowed between scaffold planks? | 1 inch |
| 5 | When must a scaffold be re-inspected? | After any modification and after any event that could affect structural integrity |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #25: Scaffold Safety — Building It Right the First Time  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years