ROOFING SAFETY — TOOLBOX TALK Scaffold Inspection — What to Look For Before You Step On Talk #26 of 30 ¦ Cal/OSHA: §1635–1655 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Scaffold Base Plate Sinks Overnight — Morning Collapse

A scaffold was erected on soil adjacent to a home in Ventura. Overnight, irrigation water softened the soil under one base plate. In the morning, a roofer climbed onto the platform without inspecting the base. The soft soil allowed one leg to sink 4 inches, causing the scaffold to tilt and collapse. He fell 11 feet. The company had no record of a morning inspection. 'We just assumed it was fine because it was fine yesterday,' the foreman told investigators.

Discussion starter: Who inspects our scaffold each morning? What do they check?

KEY SAFETY POINTS — TODAY'S TOPIC

The Daily Pre-Shift Scaffold Inspection

- Check all base plates and mud sills — must be on firm, level, dry ground

- Check all frames, cross braces, and diagonal braces for bends, cracks, or missing pins

- Check all planking for cracks, knots, splits, or excessive deflection (bouncing)

- Check all guardrails, mid-rails, and toe boards — all must be in place and secure

- Check all ties to the building — confirm they are secure and undamaged

- Check for any damage that may have occurred overnight: wind, impact, vehicle contact

Inspecting After Adverse Events

- After any high wind (over 25 mph): full re-inspection before resuming work

- After rain: check base plate stability — wet soil can shift under load

- After any impact to the scaffold: remove from service until re-inspected

- After moving or adjusting the scaffold: full re-inspection by competent person

Tagging and Documenting Inspections

- Use a scaffold inspection tag or daily log — document date, inspector name, and findings

- If defects are found: tag the scaffold 'DO NOT USE,' remove workers, and correct before returning

- Keep inspection records with the job file — Cal/OSHA may request them

- Never erase or alter inspection records — document accurately, including defects found and corrected

CAL/OSHA REQUIREMENT: Cal/OSHA §1637(e) requires scaffolds to be inspected by a competent person before each work shift and after any occurrence that could affect structural integrity, including high winds, heavy rain, or any impact.

DISCUSSION QUESTIONS — ASK YOUR CREW

- When was the last time our scaffold was formally inspected? By whom?

- What would you do if you arrived in the morning and noticed the scaffold had shifted overnight?

- Is our scaffold inspection being documented? Where are those records kept?

- Has it rained recently or been windy? Did we re-inspect after that?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | When must a scaffold be inspected? | Before each shift and after any event that could affect structural integrity |
| 2 | What should you check first at the base of a scaffold during inspection? | Base plates and mud sills — confirm firm, level, dry ground |
| 3 | What should you do if you find a defect during scaffold inspection? | Tag 'DO NOT USE,' remove workers, correct before returning |
| 4 | After heavy rain, what specific scaffold component is most at risk? | Base plate stability — wet soil can shift under load |
| 5 | Must scaffold inspections be documented? | Yes — date, inspector name, and findings must be recorded |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #26: Scaffold Inspection — What to Look For Before You Step On  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years