ROOFING SAFETY — TOOLBOX TALK Silica Dust — The Silent Killer in Old Roofing Materials Talk #28 of 30 ¦ Cal/OSHA: §5204, §5144 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Roofer Diagnosed With Silicosis at Age 38 — Lungs Permanently Scarred

A 38-year-old roofer in Bakersfield was diagnosed with silicosis — permanent, progressive scarring of the lung tissue — after 15 years of tear-off roofing. He had spent his career doing dry tear-offs of old concrete tile and built-up gravel roofs with no respiratory protection. 'Nobody ever told me the dust could do this,' he said. 'I always thought if it didn't make me cough a lot, it was fine.' Silicosis has no cure. He is now on supplemental oxygen at age 42.

Discussion starter: Have you ever done dry tear-off work on old concrete tile, gravel roofing, or built-up roofing without respiratory protection?

KEY SAFETY POINTS — TODAY'S TOPIC

Where Silica Dust Comes From in Roofing

- Concrete and clay tile roofing: contains significant crystalline silica

- Granules from old asphalt shingles: contain silica

- Built-up roofing (BUR) with pea gravel: concentrated silica source

- Mortar, caulking, and cement-based products

- Silica particles are invisible — you cannot see them, taste them, or smell them

- Dry cutting, grinding, or dry tear-off of any of these materials releases the highest concentrations

Engineering Controls — First Defense Against Silica

- Wet methods: use water to suppress dust during cutting and tear-off — most effective control

- Local exhaust ventilation: connect power tools to vacuum systems with HEPA filters

- Enclosed cab equipment: use equipment with enclosed cabs and filtered air when available

- Avoid dry sweeping: use wet methods or HEPA-filtered vacuums — never dry broom silica dust

- Engineering controls come FIRST — respirators are the last resort, not the primary protection

Respiratory Protection When Controls Are Not Enough

- Minimum for silica dust: N95 respirator — properly fit-tested

- Half-face respirator with P100 filters: required for high-exposure tasks (grinding, heavy tear-off)

- Respirators must be NIOSH-approved and fit-tested — a loose mask does nothing

- Medical evaluation required before using a tight-fitting respirator under Cal/OSHA

- Training required: donning, doffing, fit check, maintenance, and storage

CAL/OSHA REQUIREMENT: Cal/OSHA §5204 (Respirable Crystalline Silica Standard) requires employers to limit worker exposure to silica dust to 50 micrograms per cubic meter (PEL). Engineering controls must be used before respiratory protection.

DISCUSSION QUESTIONS — ASK YOUR CREW

- What materials are we tearing off on this job? Do any of them contain silica?

- What dust control measures are in place today? Wet methods? HEPA vacuums?

- Does anyone on this crew have a properly fit-tested respirator for silica work?

- Have you ever had symptoms of respiratory irritation after a tear-off job? What did you do about it?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What is the Cal/OSHA Permissible Exposure Limit (PEL) for respirable crystalline silica? | 50 micrograms per cubic meter |
| 2 | What is the most effective engineering control for silica dust during roofing tear-off? | Wet methods — using water to suppress dust |
| 3 | What is the minimum respirator class for silica dust protection? | N95 (fit-tested) |
| 4 | Is dry sweeping an acceptable cleanup method after silica-generating work? | No — use wet methods or HEPA-filtered vacuums only |
| 5 | Does silicosis have a cure? | No — it is permanent and progressive |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #28: Silica Dust — The Silent Killer in Old Roofing Materials  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years