ROOFING SAFETY — TOOLBOX TALK Asbestos Awareness — Old Roofing Can Be a Hidden Hazard Talk #29 of 30 ¦ Cal/OSHA: §5204, §5144 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Roofing Crew Tears Off Asbestos Roofing Without Testing

A roofing crew in Oakland tore off the roofing on a 1962 commercial building without requesting an asbestos survey. The built-up roofing contained chrysotile asbestos in the felts. Five workers were exposed to airborne asbestos fibers during a full day of tear-off. Cal/OSHA discovered the situation when a neighbor called the environmental hotline after seeing debris being thrown off the roof. The contractor faced criminal charges and $280,000 in fines. The five workers are now monitored annually for mesothelioma.

Discussion starter: How do you determine whether a roofing system might contain asbestos before you start tearing it off?

KEY SAFETY POINTS — TODAY'S TOPIC

When to Suspect Asbestos in Roofing Materials

- ANY roofing material on buildings constructed or last re-roofed BEFORE 1980: suspect asbestos

- Built-up roofing (BUR) felts, especially on commercial buildings pre-1980

- Vinyl/asphalt floor tiles and adhesives (also present on flat roof systems)

- Roof coating materials, caulking, and mastics on pre-1980 buildings

- The only way to confirm: testing by a Certified Asbestos Consultant (CAC)

What You Must Do Before Disturbing Suspected ACM

- Do NOT disturb the material until testing is complete

- Hire a California-certified Asbestos Consultant (CAC) to collect samples and test

- If asbestos is confirmed: only California-licensed asbestos abatement contractors can remove it

- Notify Cal/OSHA district office before any Class I asbestos work (removal of TSI or surfacing ACM)

- As a roofing contractor: your job is to STOP WORK if asbestos is suspected and call the right people

Your Legal Obligations as a Roofing Contractor

- Cal/OSHA §1529 and the Cal/EPA Asbestos regulations make unauthorized disturbance of ACM a serious violation

- Fines can exceed $100,000 per day of violation

- Criminal charges are possible for willful violation

- You cannot plead ignorance — the age of the building determines your obligation to investigate

- Document your asbestos survey process for every pre-1980 building tear-off

CAL/OSHA REQUIREMENT: Cal/OSHA §1529 requires that workers be informed about the presence of asbestos-containing materials (ACM) before work begins. Any suspected ACM must be tested by a Certified Asbestos Consultant before disturbance. Disturbing ACM without proper procedures is a serious criminal violation.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Is this building old enough that we should be thinking about asbestos?

- What is our company's procedure when we suspect asbestos in roofing materials?

- Do you know what a California Certified Asbestos Consultant is and how to find one?

- Have you ever worked on a job where asbestos was discovered after work had started? What happened?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What year is the general cutoff for suspecting asbestos in roofing materials? | Buildings constructed or last re-roofed before 1980 |
| 2 | Who must collect and test samples for asbestos before removal? | A California Certified Asbestos Consultant (CAC) |
| 3 | Can a licensed roofing contractor remove confirmed asbestos roofing materials? | No — only licensed asbestos abatement contractors |
| 4 | What should a roofing crew do if they suspect asbestos mid-job? | Stop work immediately and contact the employer and a CAC |
| 5 | What is the regulatory basis for asbestos work procedures in California construction? | Cal/OSHA §1529 |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #29: Asbestos Awareness — Old Roofing Can Be a Hidden Hazard  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years