ROOFING SAFETY — TOOLBOX TALK Respiratory Protection Program — Wearing It Right Talk #30 of 30 ¦ Cal/OSHA: §5204, §5144 ¦ Approx. 10–15 minutes

| Company: |  | Date: |  |
| --- | --- | --- | --- |
| Job Site / Location: |  | Conducted By: |  |

REAL INCIDENT — WHAT HAPPENED?

⚠ REAL INCIDENT: Worker Wears N95 Incorrectly for Years — Zero Protection

An investigation at a commercial roofing company in Fresno found that workers had been issued N95 respirators for silica-generating work for 3 years. However, several workers had not been fit-tested, and two had facial hair that prevented a proper seal. A third worker had been wearing the respirator upside down for months. Silica exposure monitoring found all three had been exposed above the PEL consistently. 'Having a respirator is not the same as being protected by a respirator,' the Cal/OSHA inspector noted.

Discussion starter: If you have a respirator, do you know how to check that it is sealing properly against your face right now?

KEY SAFETY POINTS — TODAY'S TOPIC

The Seal Check — Do This Before Every Use

- Positive pressure check: cover exhalation valve, exhale gently — the mask should puff out slightly and hold

- Negative pressure check: cover inhalation valves, inhale — the mask should collapse slightly against your face

- If air leaks around the edges: reposition, readjust, or the respirator does not fit you

- A loose respirator provides little to no protection — do not use if seal cannot be achieved

- Facial hair in the seal zone: even stubble breaks the seal — shave or use a loose-fitting respirator

Fit Testing — Required Before First Use

- All workers required to use tight-fitting respirators must be fit-tested BEFORE first use

- Fit test must be repeated: annually, when a different size/model is used, and when facial changes occur (weight loss, dental work)

- Qualitative fit test uses taste/smell agents; quantitative fit test uses measurement equipment

- Fit test must be documented — keep records with the written Respiratory Protection Program

- A fit test done by your employer at no cost to you — it is the employer's responsibility

Respirator Maintenance and Storage

- Clean respirators after each use with the manufacturer's recommended cleaning method

- Inspect for cracks, tears, and damaged valves after each cleaning

- Store in a sealed bag or container away from dust, chemicals, and direct sunlight

- Replace filters/cartridges according to the manufacturer's schedule or when breathing becomes difficult

- Disposable N95s: single-use only — do not reuse if wet, soiled, or damaged

- Never share respirators between workers — they are individually fit-tested

CAL/OSHA REQUIREMENT: Cal/OSHA §5144 requires a written Respiratory Protection Program when respirators are required. The program must include: medical evaluation, fit testing, training, and maintenance procedures for all workers required to use respirators.

DISCUSSION QUESTIONS — ASK YOUR CREW

- Has everyone on this crew who needs a respirator been properly fit-tested?

- Can you demonstrate the positive and negative pressure seal check right now?

- Does anyone have facial hair in the seal zone that could break their respirator seal?

- Where are we storing our respirators at the end of the day — are they being protected?

KNOWLEDGE CHECK QUIZ

Supervisor: Ask each question aloud. Verify crew members can answer correctly before moving on.

| # | Question | Answer |
| --- | --- | --- |
| 1 | What does a positive pressure seal check involve? | Cover the exhalation valve, exhale gently — the mask should puff out and hold |
| 2 | How often must fit testing be repeated? | Annually, and when size/model changes or facial changes occur |
| 3 | Can two workers share the same tight-fitting respirator? | No — respirators are individually fit-tested and must not be shared |
| 4 | Why does facial hair in the seal zone make a respirator ineffective? | Even stubble breaks the seal — allowing contaminated air to bypass the filter |
| 5 | Are disposable N95 respirators reusable? | No — single-use only; discard when wet, soiled, or damaged |

ATTENDANCE SIGN-IN SHEET

All employees present must sign below to confirm they attended this toolbox talk and understood the content.

| Employee Name (Print) | Signature | Date | Understood? |
| --- | --- | --- | --- |

| Supervisor Signature |  | Date |
| --- | --- | --- |

RoofSafe Pro  |  Talk #30: Respiratory Protection Program — Wearing It Right  |  Keep on file — Cal/OSHA §3203(b) requires training records be retained minimum 3 years